CSAI321 – Computer Networks

Hard Mode Contest Practice Sheet

Revised, Reshuffled, Harder MCQs — All 6 Lectures

How to use this sheet: Every question is application/calculation-heavy (mid-to-super-hard difficulty). Options are randomly shuffled per question — the correct letter is NOT clustered on any single option, so don't pattern-guess. Cover the green answer box while attempting, then check. Good luck for the test!

# Lecture 1 — How the Internet Works (Hard)

Q1. A submarine cable break near Marseille is repaired within hours because traffic reroutes through the Suez and Cape routes, but users in the region still see a temporary latency spike rather than an outage. Which single fact from the lecture BEST explains why an outage was avoided at all?

(A) The Internet is a federation of interconnected networks with no single controlling path, so packet switching allows traffic to route around a failed link

(B) TLS re-established a new session key for the alternate path

(C) AWS Edge Locations automatically absorbed the lost capacity

(D) Submarine cables are only used for backup traffic, not primary traffic

Answer: A. The Internet's resilience comes from being a decentralized network of networks combined with packet switching's ability to reroute — no single organization or single physical path controls delivery.

Q2. Which of the following correctly ranks network types from smallest to largest geographic scope AND correctly classifies the Internet itself?

(A) LAN < MAN < PAN < WAN; the Internet is a PAN

(B) PAN < LAN < MAN < WAN; the Internet is a WAN (a network of networks)

(C) PAN < MAN < LAN < WAN; the Internet is a LAN

(D) LAN < PAN < WAN < MAN; the Internet is a MAN

Answer: B. Correct scope order is PAN → LAN → MAN → WAN, and the Internet, spanning the globe as an interconnection of many networks, is best classified as a WAN.

Q3. An AWS architect deploys an application across two Availability Zones inside the Mumbai Region, and also enables an Edge Location cache for static assets. A fire disables one AZ's power completely. Which statement is FALSE?

(A) The Edge Location cache is what keeps the application running during the AZ outage

(B) The Region itself is unaffected structurally — only one of its data-center clusters lost power

(C) The application likely stays online because the other AZ is an isolated fault domain

(D) This scenario illustrates why multi-AZ deployment is described as fault tolerant

Answer: A. Edge Locations reduce latency via caching/proximity — they don't provide compute failover. Surviving the outage is due to the multi-AZ deployment, not the CDN cache.

Q4. Two threats to submarine cables were highlighted in the lecture. Besides fishing anchors/trawlers, what was the other (illustrative) threat named?

(A) Earthquakes

(B) Illegal tapping by governments

(C) Corrosion from saltwater

(D) Sharks

Answer: D. The notes specifically cite fishing anchors/trawlers as the primary cause and sharks as a secondary, semi-humorous but real threat.

Q5. In the UCLA-to-SRI 1960s origin story used as a networking trivia anchor, what actually happened?

(A) A file transfer completed but was corrupted

(B) The connection never got established at all

(C) The word 'login' was being sent, but the system crashed after only 'lo' got through

(D) The word 'hello' was sent successfully in full

Answer: C. Only the letters 'lo' of 'login' made it through before the system crashed — the classic ARPANET origin anecdote.

Q6. Which pairing of an AWS concept to its correct role is INCORRECT?

(A) Availability Zone — one or more discrete, isolated-fault-domain data centers within a Region

(B) VPC — a private, software-defined network where you control IP ranges and subnets

(C) Region — an isolated geographic area with multiple data centers

(D) Edge Location — provides your own private, IP-range-controlled network inside AWS

Answer: D. That description belongs to a VPC, not an Edge Location. Edge Locations are CDN endpoints that cache content near users to cut propagation delay.

Q7. A student says: 'Tapping the Instagram icon basically just means WiFi talks to the Instagram app.' What is the main conceptual error in this statement, per the lecture?

(A) Instagram does not use HTTP requests at all

(B) WiFi is never involved in mobile app requests

(C) The statement is actually completely correct as stated

(D) It collapses a multi-hop journey (device → router → ISP → core → data center) into a two-step interaction, hiding the real complexity

Answer: D. The lecture explicitly uses this as an example of oversimplification — the real journey has many intermediate networking components most users never think about.

Q8. Which THREE organizations are named as the major cloud providers whose networks the course references?

(A) AWS, Oracle Cloud, IBM Cloud

(B) Azure, GCP, DigitalOcean

(C) AWS, Azure, Cloudflare

(D) AWS, Microsoft Azure, Google Cloud Platform

Answer: D. The notes explicitly name AWS, Azure, and GCP as the three cloud providers discussed.

Q9. A GPU training cluster uses 200 Gbps InfiniBand between nodes. Why does the lecture bring this up in a networking course rather than a hardware course?

(A) To prove that InfiniBand has replaced TCP/IP entirely

(B) To argue that GPUs no longer need networking once training starts

(C) To show that even AI infrastructure fundamentally depends on high-speed networking to move data between compute nodes

(D) To demonstrate that Ethernet is faster than InfiniBand

Answer: C. The point is that networking is the backbone even of AI/GPU infrastructure — fast interconnects like InfiniBand exist because compute nodes must constantly exchange data.

Q10. Which statement about 'who controls the Internet' is most accurate per the lecture?

(A) The UN has final authority over global routing

(B) Each country's government fully controls its own portion with no external interconnection

(C) AWS, Azure, and GCP jointly govern all Internet traffic

(D) No single organization controls it; it is a federation of countless independent networks (ISPs, companies, governments)

Answer: D. The Internet is explicitly described as a federation with no single controlling entity, even though individual networks are independently owned/operated.

# Lecture 2 — Moving Data Through the Core (Hard / Calculation-heavy)

Q11. A 15,000-bit packet is sent over a link with rate 3 Mbps, across a distance of 9×10^5 m with signal speed 3×10^8 m/s. What is the TOTAL of transmission + propagation delay (ignoring processing/queuing)?

(A) 8 ms (5 ms transmission + 3 ms propagation)

(B) 5 ms (all transmission, propagation is negligible)

(C) 11 ms (6 ms transmission + 5 ms propagation)

(D) 3 ms (transmission only)

Answer: A. Transmission = L/R = 15000/3,000,000 = 5 ms. Propagation = d/s = 900,000/300,000,000 = 3 ms. Total = 8 ms.

Q12. A path has four links in series: 40 Mbps, 1 Gbps, 25 Mbps, and 200 Mbps. A separate parallel path exists only for control messages and is irrelevant to data throughput. What is the approximate end-to-end DATA throughput, and which link should be upgraded first for the biggest real-world gain?

(A) 25 Mbps end-to-end; upgrade the 25 Mbps link first since it's the bottleneck

(B) 40 Mbps end-to-end; upgrade the 40 Mbps link first

(C) 316 Mbps (the average); upgrade the 1 Gbps link for the biggest gain

(D) 1 Gbps end-to-end since that's the fastest link available

Answer: A. Throughput ≈ min(all link rates) = 25 Mbps. Upgrading any link that isn't the bottleneck (e.g., the already-fast 1 Gbps link) has zero effect on end-to-end throughput.

Q13. If you simultaneously double the link rate R and double the packet size L, what happens to the transmission delay?

(A) It quadruples

(B) It halves

(C) It doubles

(D) It stays exactly the same, since L/R is unchanged when both double

Answer: D. dtrans = L/R. Doubling both numerator and denominator leaves the ratio unchanged.

Q14. Which statement about queuing delay, as scoped for this exam, is correct?

(A) It is always exactly equal to processing delay

(B) It never occurs in real networks, only in theory

(C) It is real and variable (buffer-dependent), but its mathematics is explicitly out of exam scope

(D) It is calculated with the formula d/s, same as propagation delay

Answer: C. Queuing delay is conceptually real (congestion-dependent) but the notes explicitly exclude its math from the exam — only L/R and d/s calculations are examinable.

Q15. A network engineer claims: 'Since packet switching has no reservation, statistical multiplexing gives us efficiency AND resilience simultaneously.' Which pairing correctly matches each benefit to its mechanism?

(A) Efficiency comes from links being shared only when actually needed (bursty traffic rarely aligns); resilience comes from packets not being tied to one fixed path

(B) Both efficiency and resilience come purely from higher link rates

(C) Efficiency comes from guaranteed reserved bandwidth; resilience comes from store-and-forward

(D) Efficiency comes from rerouting; resilience comes from statistical sharing

Answer: A. Efficiency: links aren't reserved, so unused capacity during a burst-free moment can serve other users. Resilience: since nothing is permanently reserved on one path, traffic can reroute if a link/router fails.

Q16. A satellite link advertises 1 Gbps bandwidth. A geostationary satellite orbits at ~35,786 km, giving a round-trip distance of ~71,500 km at signal speed ~2×10^8 m/s (fiber-like handling on the ground segments). Roughly what round-trip propagation delay should you still expect, REGARDLESS of the 1 Gbps rating?

(A) Roughly 1 ms, because 1 Gbps is a very high bandwidth

(B) Zero, because high bandwidth links have no propagation delay

(C) It's impossible to estimate without knowing the packet size

(D) Roughly 240 ms — propagation delay depends on distance and speed, not on bandwidth

Answer: D. Propagation delay = d/s regardless of bandwidth. The 71,500 km round trip at roughly 3×10^8 m/s produces about 240 ms — a physical limit no amount of bandwidth removes.

Q17. Which of the following is NOT one of the four delay components a packet experiences at a node?

(A) Processing delay

(B) Transmission delay

(C) Encryption delay

(D) Queuing delay

Answer: C. The four delays are Processing, Queuing, Transmission, and Propagation. Encryption delay isn't one of the four nodal-delay components covered.

Q18. A router receives 70% of a packet's bits when the incoming link abruptly fails. Per store-and-forward, what happens, and what does this reveal about a router's required knowledge of the path?

(A) The router buffers indefinitely, waiting for the link to recover

(B) The router requests the missing 30% directly from the original sender using ICMP retransmission

(C) The router forwards the 70% it has, since 'something is better than nothing'

(D) The partial packet is discarded (never forwarded), which shows routers only need to know the next hop, not the full end-to-end path

Answer: D. A router must fully receive a packet before forwarding any of it; an incomplete packet is not forwarded. This is consistent with the router only ever needing to know the next hop.

Q19. A landline call and a WhatsApp call both go silent for 10 seconds mid-conversation. What is the KEY difference in what happens to network capacity during that silence?

(A) Landline: the reserved capacity sits idle and unusable by others; WhatsApp: no packets are sent, so that capacity becomes available for others

(B) The landline call automatically reroutes through packet switching during silence

(C) WhatsApp reserves capacity in advance just like the landline

(D) Both waste capacity identically since both used a network

Answer: A. Circuit switching reserves a dedicated path for the whole session regardless of activity; packet switching only uses the link when packets are actually being sent.

Q20. A packet is 20,000 bits, sent over a 4 Mbps link, distance 4×10^5 m, signal speed 2×10^8 m/s. Which of the delays is LARGER, and by how much?

(A) Transmission delay is larger — 5 ms vs 2 ms propagation (3 ms more)

(B) Transmission delay is larger — 8 ms vs 2 ms propagation (6 ms more)

(C) Propagation delay is larger — 5 ms vs 2 ms transmission (3 ms more)

(D) They are exactly equal at 4 ms each

Answer: A. dtrans = 20000/4,000,000 = 5 ms. dprop = 400,000/200,000,000 = 2 ms. Transmission is larger by 3 ms.

Q21. Why is packet switching described in the notes as 'the seed of everything TCP does'?

(A) Because TCP only works with circuit-switched networks

(B) Because packet switching guarantees delivery, making TCP's job trivial

(C) Because the network core is best-effort/unreliable, TCP must handle loss detection, retransmission, and pacing to make it feel reliable

(D) Because TCP was invented before packet switching existed

Answer: C. Because packet switching offers no delivery guarantees, TCP at the transport layer must compensate — detecting loss, retransmitting, and pacing data.

Q22. A 100 Mbps home plan actually delivers only 12 Mbps during peak hours for a Netflix stream. Which explanation is MOST consistent with the throughput/bottleneck concept from the lecture?

(A) Some other hop along the full path (a congested peering link, an overloaded server, or home WiFi) is the true bottleneck, not the advertised 100 Mbps access link

(B) 12 Mbps is simply what Netflix always throttles regardless of network conditions

(C) The 100 Mbps figure was always false advertising with no technical basis

(D) Throughput is unrelated to any single link and is purely a function of packet size

Answer: A. Your ISP plan is just one hop; end-to-end throughput is bounded by the slowest link anywhere on the path — the true question is always 'which hop is the slowest?'

Q23. Which statement about packet anatomy and delivery independence is correct?

(A) Packets in packet switching require the full path to be pre-established before any packet is sent

(B) A packet = header (addressing) + payload (data), and different packets from the same message can take different routes to the same destination

(C) All packets from one message are guaranteed to follow the identical physical route

(D) A packet only contains payload; headers are added exclusively by the application layer at the very end

Answer: B. Packets are independently routable header+payload units; this independence is precisely why packet switching doesn't need pre-reservation.

# Lecture 3 — OSI & TCP/IP Layering (Hard)

Q24. A packet crosses from a laptop in Network A, through Network B, to a server in Network C. Which statement about Layer 2 vs Layer 3 behavior during this journey is correct?

(A) Layer 3 (IP) addressing largely persists end-to-end across the journey, while Layer 2 (MAC) framing is stripped and rebuilt fresh at every hop/network boundary

(B) Layer 2 addressing persists end-to-end while Layer 3 addressing is rebuilt at every hop

(C) Both Layer 2 and Layer 3 addressing stay identical for the entire journey

(D) Neither layer's addressing survives even a single hop

Answer: A. Layer 3 represents the whole journey (source/destination IP largely unchanged), while Layer 2 is strictly one-hop scope — a new frame with new MAC addresses is built at every network boundary.

Q25. Why would it be a design catastrophe for a Layer 3 router to try to read and interpret Layer 7 application data on every packet it forwards?

(A) It would only slightly slow things down with no other consequences

(B) Routers are physically incapable of reading any data beyond Layer 1 bits

(C) It would require understanding every possible application protocol, massively slow down forwarding via deep inspection, and break the encryption/privacy model

(D) It's actually a common and encouraged practice in real routers today

Answer: C. Layering's entire purpose is that a device only needs intelligence up to the layer it operates at; violating this breaks scalability, speed, and privacy (especially with TLS encryption in the way).

Q26. Match the PDU (Protocol Data Unit) names to their correct layer, from Transport down to Physical.

(A) Frame (Transport) → Packet (Network) → Segment (Data Link) → Bits (Physical)

(B) Segment (Transport) → Frame (Network) → Packet (Data Link) → Bits (Physical)

(C) Packet (Transport) → Segment (Network) → Frame (Data Link) → Bits (Physical)

(D) Segment (Transport) → Packet (Network) → Frame (Data Link) → Bits (Physical)

Answer: D. Transport = Segment, Network = Packet, Data Link = Frame, Physical = Bits.

Q27. In the gift-wrapping encapsulation analogy, why can't you remove the inner wrapping before the outer wrapping?

(A) It mirrors decapsulation order: each layer strips only its own outermost header first, moving strictly bottom-to-top as data ascends the stack

(B) Because encryption prevents any layer from being opened out of order

(C) Because the sender chooses a random unwrapping order each time

(D) It's just a physical constraint of paper and ribbon with no networking parallel

Answer: A. Encapsulation/decapsulation must happen in strict reverse-order symmetry — you can't decapsulate Layer 3's header before Layer 4's header has been removed on the way up.

Q28. The TCP/IP model's 'Application' layer maps to which OSI layers, and why does this matter practically?

(A) Only OSI Layer 7 — the other layers have no TCP/IP equivalent at all

(B) OSI Layer 4 only, since Transport and Application are considered identical in TCP/IP

(C) OSI Layers 1 through 4 — TCP/IP inverts the OSI stack

(D) OSI Layers 5, 6, and 7 (Session, Presentation, Application) — TCP/IP is the model the Internet actually runs on, while OSI remains the teaching/conceptual model

Answer: D. TCP/IP's condensed 4-layer model folds OSI's Session, Presentation, and Application layers into a single Application layer.

Q29. A vendor swaps a company's WiFi access points for Ethernet cabling without needing to modify any application code. Which layering benefit does this best illustrate?

(A) Store-and-forward

(B) Encapsulation symmetry

(C) Layer replaceability — each layer can be substituted independently as long as the same interface to adjacent layers is preserved

(D) Statistical multiplexing

Answer: C. One of the four core benefits of layering is that you can replace a lower layer's technology (WiFi ↔ Ethernet) without rewriting the layers above it.

Q30. Which layer is responsible for 'Media Access Control' — deciding who transmits when multiple devices share a medium — and what OTHER responsibility does that same layer carry?

(A) Layer 4 (Transport); it also performs media access control and encryption

(B) Layer 1 (Physical); it also performs IP routing

(C) Layer 2 (Data Link); it also performs error checking on received frames

(D) Layer 3 (Network); it also performs MAC address learning

Answer: C. Layer 2 (Data Link) handles both media access control (avoiding simultaneous transmission) and error checking (verifying bits received = bits sent).

Q31. Which of these is the CORRECT top-down OSI mnemonic AND its correct layer order (Layer 7 to Layer 1)?

(A) 'Please Do Not Throw Sausage Pizza Away' → Physical, Data Link, Network, Transport, Session, Presentation, Application

(B) 'Please Do Not Throw Sausage Pizza Away' → Application, Presentation, Session, Transport, Network, Data Link, Physical

(C) 'All People Seem To Need Data Processing' → Application, Presentation, Session, Transport, Network, Data Link, Physical

(D) 'All People Seem To Need Data Processing' → Physical, Data Link, Network, Transport, Session, Presentation, Application

Answer: C. 'All People Seem To Need Data Processing' is top-down (L7→L1); 'Please Do Not Throw Sausage Pizza Away' is the bottom-up mnemonic (L1→L7).

Q32. Why does a switch never need to understand IP addresses to do its job correctly?

(A) Because a switch broadcasts every frame to every port regardless of address

(B) Because switches are actually Layer 4 devices that use port numbers instead

(C) Because a switch operates at Layer 2 and forwards frames using learned MAC-address-to-port mappings, which is a one-LAN, one-hop scope of operation

(D) Because IP addresses don't exist inside any LAN

Answer: C. Switches operate at Layer 2 (Data Link), which only requires MAC-address awareness within a single LAN — Layer 3 (IP) is out of scope for a switch's job.

Q33. A developer says: 'Encapsulation just means adding random extra bytes to a message.' What is the precise correction, based on the 'private conversation' idea from the lecture?

(A) Encapsulation means each layer at the sender adds ITS OWN header, which only the matching layer at the receiver reads and strips — layers never touch each other's headers

(B) Encapsulation is only relevant to Layer 7 and doesn't apply to lower layers

(C) Encapsulation means every layer reads and rewrites every other layer's header for redundancy

(D) Encapsulation removes headers rather than adding them

Answer: A. Each layer's header is a 'private conversation' with the same layer on the receiving end — e.g., only the receiver's Layer 3 reads what the sender's Layer 3 wrote, preserving the layering abstraction.

# Lecture 4 — Devices, Topologies & Cloud (Hard)

Q34. A company grows from 20 to 200 employee devices and insists on full-mesh direct cabling (no central device). Approximately how many MORE cables are needed at 200 devices compared to 20 devices?

(A) 200 − 20 = 180 additional cables

(B) 19,900 − 190 = 19,710 additional cables

(C) No additional cables needed if a switch replaces some connections

(D) Exactly double, since devices doubled... tripled... (10x devices = 10x cables)

Answer: B. n(n-1)/2 at 20 devices = 190; at 200 devices = 19,900. The difference is 19,710 — illustrating the quadratic (not linear) growth in cabling need.

Q35. A basic packet-filtering firewall has the rule: 'Allow only if Destination Port = 22 AND Source IP = 10.0.0.7.' A packet arrives with Source IP 10.0.0.7 and Destination Port 443. What happens, and why?

(A) Allowed — since the source IP matches, the port doesn't matter

(B) Allowed — basic packet filtering only checks source IP, never ports

(C) Blocked — the AND condition requires BOTH to match, and the port (443 ≠ 22) fails

(D) Blocked — because 10.0.0.7 is never a valid source IP

Answer: C. Basic packet filtering checks source/destination IP AND source/destination port; ALL specified conditions must hold. Here the port mismatch alone causes a block.

Q36. Which single sentence BEST explains why network devices differ from one another (hub vs switch vs router vs firewall vs load balancer)?

(A) They differ only in the era they were invented, with no functional difference

(B) They differ mainly in how deeply into the packet they inspect / which OSI layer they operate at, which determines how 'intelligent' their forwarding decision can be

(C) They all operate at Layer 1 but use different cabling standards

(D) They differ only in physical size and price, not in function

Answer: B. The notes frame this explicitly: devices operate at different OSI layers and differ mainly in how deeply they inspect packets, which determines the sophistication of the forwarding/filtering decision.

Q37. A smart-home installer is choosing between Star and Mesh topology for 15 IoT devices (bulbs, cameras, sensors) that need to stay online even if one device fails. Using the fault-tolerance-vs-scalability trade-off table from the lecture, what's the correct call?

(A) Hybrid — because it's always the objectively best choice regardless of context

(B) Ring — because a single break doesn't affect the rest of a ring topology

(C) Mesh — high fault tolerance and multiple redundant device-to-device paths fit reliability-critical IoT devices, even though cabling/connections scale worse

(D) Star — because it has 'good' scalability and that matters more than fault tolerance for smart homes

Answer: C. Per the notes' environment-to-topology mapping, Mesh suits smart homes precisely because of its high fault tolerance from redundant direct paths, even though hybrid is typical for larger data centers.

Q38. Which claim about the relationship between traditional physical networking and cloud networking is FALSE?

(A) A load balancer in AWS still performs conceptually the same job as a physical one

(B) Cloud networking makes concepts like load balancers and subnets obsolete since everything is now virtual

(C) API calls/console clicks replace manual physical wiring

(D) A VPC is the cloud analog of your own private, controllable network space

Answer: B. The notes explicitly state cloud networking doesn't make prior concepts obsolete — it changes HOW you instantiate them (software-defined), not WHAT the concepts are.

Q39. A firewall inspecting only IP and port information is said to read 'two layers deep.' Which two layers, specifically?

(A) Layer 1 (bits) and Layer 2 (frames)

(B) Layer 3 (IP addresses) and Layer 4 (port numbers)

(C) Layer 2 (MAC addresses) and Layer 3 (IP addresses)

(D) Layer 4 (ports) and Layer 7 (application content)

Answer: B. IP addresses belong to Layer 3 and port numbers belong to Layer 4, so a basic packet-filtering firewall inspects exactly those two layers.

Q40. 6 incoming requests arrive at a service backed by 3 identical servers with a load balancer. If server capacity — not network bandwidth — is the actual bottleneck, what is the CORRECT fix per the lecture's framing?

(A) Upgrade the network link bandwidth, since that's always the bottleneck

(B) Add more servers and let the load balancer distribute traffic — 'stop sending everything to one server' rather than 'make one server faster'

(C) Switch the servers from packet switching to circuit switching

(D) Remove the load balancer to reduce latency overhead

Answer: B. The lecture explicitly frames load balancing as the fix for a server-side (not link-side) bottleneck: distribute load across more servers rather than trying to speed up a single one.

Q41. Devices by OSI layer: which single ordering below correctly matches Hub → Switch → Router → Firewall → Load Balancer to their layers/decision basis?

(A) L2 (MAC) → L1 (broadcast) → L4 (port) → L3 (IP) → L7 (server choice)

(B) L1 (nothing/broadcast) → L2 (MAC) → L3 (IP) → L3/L4 (IP+Port) → L4/L7 (server choice)

(C) L4 (port) → L3 (IP) → L2 (MAC) → L1 (broadcast) → L7 (server choice)

(D) L1 (broadcast) → L3 (IP) → L2 (MAC) → L4 (port) → L3/L4 (server choice)

Answer: B. This is the 'single most important table' from the lecture: Hub=L1, Switch=L2 (MAC), Router=L3 (IP), Firewall=L3/L4 (IP+Port), Load Balancer=L4/L7 (server selection).

Q42. Why is a hub described as fundamentally 'wasteful' compared to a switch, even though both can physically connect the same number of devices in a Star layout?

(A) A hub blindly broadcasts every incoming signal to all ports, forcing every other device to receive and discard irrelevant traffic, unlike a switch which learns MAC-to-port mappings

(B) A hub operates at a higher OSI layer, making it slower

(C) A hub cannot be used in a Star topology at all

(D) A hub uses more physical cabling than a switch for the same number of devices

Answer: A. The hub's core weakness is broadcasting to every port regardless of the intended recipient, unlike a switch ('a hub with memory') which forwards intelligently.

Q43. An AWS customer wants their app to survive a full data-center power loss within one geographic area, AND wants low-latency delivery of static images to users worldwide. Which TWO AWS concepts most directly address these two separate goals respectively?

(A) VPC (fault tolerance) and Region (latency)

(B) Load Balancer (fault tolerance) and VPC (latency)

(C) Multi-AZ deployment (fault tolerance) and Edge Locations (latency via caching)

(D) Subnets (fault tolerance) and Availability Zones (latency)

Answer: C. Multi-AZ deployment protects against a single data center's failure (isolated fault domains); Edge Locations cache content physically closer to users, cutting propagation delay.

# Lecture 5 — HTTP, P2P & REST (Hard)

Q44. A frontend developer wants to update ONLY a user's email field without risking accidental deletion of the rest of the user's profile data. Which method should be used, and precisely why is the alternative risky?

(A) GET — because GET can carry a request body with the new field

(B) PUT — because PATCH is deprecated in modern REST APIs

(C) DELETE followed by POST — since PATCH doesn't exist in the HTTP spec

(D) PATCH — because PUT semantically means 'replace the entire resource,' so a partial PUT payload could wipe out unlisted fields

Answer: D. PUT means full replacement; sending only the email field via PUT could be interpreted as replacing the whole resource with just that field. PATCH is designed exactly for partial updates.

Q45. A page has a slow hero image plus a CSS file, JS file, and icon. Under HTTP/2, the CSS/JS/icon are no longer blocked behind the slow image at the APPLICATION level — yet a real stall can still occur. Why?

(A) Because HTTP/2 removes binary framing, causing parsing delays

(B) Because HTTP/2 uses more TCP connections than HTTP/1.1, increasing collision risk

(C) Because HTTP/2 still runs over one TCP connection, so a single lost TCP segment can stall ALL multiplexed streams (TCP-level HoL blocking)

(D) Because HTTP/2 doesn't support multiplexing at all

Answer: C. HTTP/2 fixes application-level HoL blocking via multiplexing over ONE TCP connection, but a lost segment on that single connection can still stall every stream — this is TCP-level HoL blocking, only solved by HTTP/3's QUIC.

Q46. Which HTTP methods are considered SAFE to use with HTTP/1.1 pipelining, and precisely why are the others excluded?

(A) DELETE and PATCH — because they are idempotent by definition

(B) All HTTP methods equally — pipelining has no restrictions

(C) POST and PUT only — because GET cannot be pipelined under any circumstances

(D) GET, HEAD, and OPTIONS — because POST/PUT can have side effects that alter server state, which is unsafe if requests are queued without waiting for prior responses

Answer: D. Only safe (read-only, no-side-effect) methods — GET, HEAD, OPTIONS — are pipeline-safe; POST/PUT are excluded because queuing state-changing requests without confirmed ordering is risky.

Q47. A shopping cart persists across pages despite HTTP being stateless. Which analogy correctly parallels this mechanism to a concept from the TLS lecture?

(A) A session cookie is like a TLS certificate: both are issued by a Certificate Authority

(B) A session cookie has no meaningful parallel to anything in TLS

(C) A session cookie is like a MAC address: both identify a device at Layer 2

(D) A session cookie is like a TLS session key: both let a one-time setup (login / handshake) be reused across many subsequent stateless exchanges without repeating it

Answer: D. Both mechanisms let an expensive one-time process (login, or the asymmetric handshake) be replaced by a reusable token/key for all following interactions, avoiding repetition.

Q48. Why does the lecture describe Client-Server as centralized and P2P as decentralized in terms of PRIVILEGE, not just architecture?

(A) Privilege has nothing to do with the client-server vs P2P distinction

(B) In Client-Server, clients depend on always-on servers with no equal standing; in P2P, every peer has equal privileges and can act as both client and server

(C) In Client-Server, every device has equal privileges; in P2P, one peer is always in charge

(D) Both architectures give every device identical privileges by design

Answer: B. Client-Server centralizes control and capability in the server; P2P decentralizes it so all peers have equal standing and dual client/server roles.

Q49. A REST API returns HTTP 204 for a successful DELETE request. What does this status code combination communicate, precisely?

(A) The delete succeeded (2xx = success) and there is no body content to return (204 = No Content)

(B) The client sent a malformed request that the server partially processed

(C) The server redirected the delete request elsewhere

(D) The delete failed silently with no error reported

Answer: A. 204 No Content is a 2xx success code that explicitly indicates the request succeeded but there's no response body to send back — common for successful DELETE operations.

Q50. Which of the following is the MOST precise description of what 'stateless' means for HTTP, distinguishing it from a total lack of any user experience continuity?

(A) Stateless means requests must always be sent in a fixed order

(B) The protocol/server does not automatically remember previous requests; any continuity (like a cart) must be engineered externally via cookies/tokens, not via HTTP itself

(C) Stateless means no data can ever be stored anywhere in the system

(D) Stateless means the server crashes after every single request

Answer: B. Statelessness is specifically about HTTP not inherently remembering prior requests — external mechanisms like cookies/sessions are layered on top to simulate continuity.

Q51. HTTP/3 replaces TCP with QUIC over UDP. Which claim about the CONSEQUENCE of this swap is correct?

(A) QUIC forces every request to open a brand-new connection, like HTTP/1.0

(B) QUIC makes HTTP stateful by default, removing the need for cookies

(C) Loss in one QUIC stream stalls only that stream, since streams are truly independent at the transport level — unlike TCP where one lost segment can stall everything multiplexed on it

(D) QUIC eliminates the need for any encryption, since UDP is inherently more secure than TCP

Answer: C. QUIC's key architectural win is per-stream loss isolation — unlike TCP's single ordered byte-stream, so one stream's loss doesn't block unrelated streams.

Q52. A developer pipelines several HTTP/1.1 requests to the same server. Even though responses come back in the CORRECT FIFO order as required, users still perceive a slowdown. What is the most likely explanation, consistent with the HoL blocking concept?

(A) One slow response at the front of the queue is still blocking every response behind it, even though the eventual order is technically correct

(B) HTTP/1.1 pipelining requires QUIC support to function

(C) FIFO ordering was violated, causing a protocol error

(D) Pipelining automatically doubles the round-trip time for every request

Answer: A. HoL blocking persists under pipelining precisely BECAUSE responses must stay in FIFO order — one slow response still delays everything queued behind it, even without violating the order requirement.

Q53. Given the CRUD ↔ HTTP method mapping, which pairing is INCORRECT?

(A) PUT = Update/full replace

(B) DELETE = partial update of a resource

(C) POST = Create

(D) GET = Read

Answer: B. DELETE removes a resource entirely; PATCH — not DELETE — is the partial update method. The other three pairings (POST=Create, GET=Read, PUT=Replace) are correct.

# Lecture 6 — HTTPS & TLS (Hard)

Q54. An attacker at a fake public WiFi hotspot forwards a self-signed certificate for 'yourbank.com' during a MITM attempt. Precisely WHERE in the TLS handshake does this attack get blocked, and why?

(A) It cannot be blocked once the attacker controls the WiFi hotspot

(B) At ClientHello (Step 1) — the browser refuses to even send a nonce to an untrusted network

(C) At Key Exchange (Step 4) — asymmetric encryption itself detects the attacker automatically

(D) At Certificate Validation (Step 3) — the browser checks the certificate chain against trusted CAs, and a self-signed cert isn't signed by any pre-installed trusted root CA, so validation fails

Answer: D. Certificate Validation is the specific step that checks whether a trusted Certificate Authority actually signed the presented certificate for that domain — a self-signed impostor cert fails this check, triggering a 'Not Secure' warning.

Q55. Why does TLS combine asymmetric AND symmetric encryption instead of using just one type throughout the whole session?

(A) Symmetric encryption is used only for the handshake, and asymmetric for all actual data

(B) Asymmetric safely solves the key-distribution problem for strangers during the handshake, while symmetric is fast enough for bulk/ongoing data — using only one would sacrifice either security-at-setup or speed-at-scale

(C) Asymmetric is used for the whole session because it is faster than symmetric encryption

(D) The combination exists purely for historical/legacy reasons with no technical benefit

Answer: B. Symmetric alone can't safely establish a shared secret between strangers (key-distribution problem); asymmetric alone is too slow for bulk data. TLS uses asymmetric for the handshake and symmetric for the fast ongoing exchange.

Q56. The browser padlock icon appears for a phishing site with a valid, CA-signed certificate for a look-alike domain (e.g., a homoglyph of a bank's real domain). What does the padlock guarantee in this case, and what does it NOT guarantee?

(A) It guarantees the site is completely safe and free of malicious intent

(B) It guarantees the site is the SAME entity as the real bank, regardless of domain spelling

(C) It guarantees the connection is encrypted and that the CERTIFICATE'S domain identity was verified — it does NOT guarantee the site itself (or its owner's intent) is trustworthy

(D) It guarantees nothing at all — padlocks are purely cosmetic

Answer: C. A CA verifies that the certificate matches the domain it was issued for — it does not vouch for whether that domain (even if legitimately purchased) is operated by a trustworthy or malicious party.

Q57. Rank the correct order of the TLS handshake steps, and identify which single step is MOST directly responsible for preventing server impersonation.

(A) ServerHello → ClientHello → Key Exchange → Certificate Validation → Encrypted Communication; Key Exchange prevents impersonation

(B) ClientHello → ServerHello+Certificate → Certificate Validation → Key Exchange → Encrypted Communication; Certificate Validation prevents impersonation

(C) ClientHello → Key Exchange → ServerHello+Certificate → Certificate Validation → Encrypted Communication; Encrypted Communication prevents impersonation

(D) Certificate Validation → ClientHello → ServerHello+Certificate → Encrypted Communication → Key Exchange; ClientHello prevents impersonation

Answer: B. The correct order is ClientHello → ServerHello+Certificate → Certificate Validation → Key Exchange → Encrypted Communication, and Certificate Validation (via CA chain of trust) is what specifically blocks impersonation.

Q58. Which statement correctly distinguishes the THREE cryptography goals (Confidentiality, Integrity, Authentication)?

(A) Confidentiality = verified identity; Integrity = no eavesdropping; Authentication = no tampering

(B) Confidentiality = no eavesdropping (achieved via encryption); Integrity = no tampering (achieved via hashing/MACs); Authentication = verified identity (achieved via certificates)

(C) Integrity = verified identity; Authentication = no tampering; Confidentiality = no eavesdropping

(D) All three goals are achieved exclusively through encryption, with no distinct mechanisms

Answer: B. Each goal maps to a distinct mechanism: Confidentiality→encryption (no eavesdropping), Integrity→hashing/MACs (no tampering), Authentication→certificates (verified identity).

Q59. A Certificate Authority's root certificate is compromised and used to sign a fraudulent certificate for a major bank. Why is this catastrophic for the ENTIRE 'chain of trust' model, not just for that one bank?

(A) Because it would only affect users who manually installed that CA's certificate

(B) Because chain of trust doesn't rely on root CAs at all, so this wouldn't matter

(C) Because browsers/OSes pre-trust root CAs by default, so any certificate signed by a compromised root would pass validation for ANY domain, not just the bank

(D) Because TLS would automatically detect and reject any compromised CA in real time with no further action needed

Answer: C. The entire chain of trust rests on browsers/OSes implicitly trusting pre-installed root CAs — if a root is compromised, an attacker can forge valid-looking certificates for ANY domain, not just one target.

Q60. Which of these correctly explains why symmetric encryption ALONE is problematic for two strangers who have never communicated before?

(A) Symmetric keys expire automatically after one use, making them unusable for strangers

(B) Symmetric encryption requires a Certificate Authority to function at all

(C) Symmetric encryption cannot mathematically encrypt data sent between two different IP addresses

(D) They have no prior secure channel to exchange the SAME secret key without risking interception — the key-distribution problem

Answer: D. Symmetric encryption needs both parties to already share the identical secret key — strangers have no safe way to exchange that key without an eavesdropper potentially intercepting it, hence the key-distribution problem.

Q61. In the HTTPS layering stack, TLS sits between which two layers, and what does that positioning enable?

(A) Below the Physical layer, encrypting raw electrical signals

(B) Between the Application layer (HTTP) and Transport layer (TCP) — allowing TLS to encrypt/authenticate application data before it's handed to TCP for reliable delivery

(C) Above the Application layer, replacing HTTP entirely

(D) Between the Network layer (IP) and Data Link layer — allowing TLS to encrypt raw frames

Answer: B. TLS sits between Application (HTTP) and Transport (TCP): HTTP data is encrypted/authenticated by TLS before being handed down to TCP for delivery.

Q62. Why is asymmetric encryption described as unsuitable for encrypting large volumes of REAL, ongoing application data (e.g., a video stream)?

(A) Asymmetric encryption doesn't support any form of confidentiality

(B) Public keys can only encrypt text, never binary streams like video

(C) It's mathematically impossible to use asymmetric encryption on more than one message

(D) It's comparatively slow, so it's reserved for the handshake/key-exchange phase rather than bulk data encryption

Answer: D. Asymmetric encryption's computational cost makes it far slower than symmetric — hence TLS uses it only to safely establish a session key, then switches to fast symmetric encryption for actual data.

# Numerical Challenge — Multi-Step Calculations

Q63. A 24,000-bit packet travels over a path with two links in series: Link A (rate 4 Mbps, distance 6×10^5 m) and Link B (rate 6 Mbps, distance 4×10^5 m), signal speed 2×10^8 m/s on both. Ignoring queuing/processing, what is the TOTAL transmission delay across BOTH links combined?

(A) 6 ms (only Link A counted, since it's slower)

(B) 24 ms (sum of packet size divided by combined rate)

(C) 2 ms (using only the faster link's rate)

(D) 10 ms (6 ms on Link A + 4 ms on Link B)

Answer: D. Link A: 24000/4,000,000 = 6 ms. Link B: 24000/6,000,000 = 4 ms. Total transmission delay = 6+4 = 10 ms (each link's transmission is separately incurred as the packet is re-transmitted at each hop).

Q64. For the same two-link path (Link A: 6×10^5 m; Link B: 4×10^5 m; both s=2×10^8 m/s), what is the TOTAL propagation delay across both links?

(A) 10 ms, same as the transmission delay total

(B) 1 ms total, since propagation is always negligible

(C) 3 ms, counting only the longer link

(D) 5 ms (3 ms on Link A + 2 ms on Link B)

Answer: D. Link A: 600,000/200,000,000 = 3 ms. Link B: 400,000/200,000,000 = 2 ms. Total = 5 ms.

Q65. A path has links rated 50 Mbps, 8 Mbps, and 120 Mbps in series. A 16,000-bit packet needs to be transmitted across the FIRST link only. What is (a) the transmission delay on the first link and (b) the overall end-to-end throughput of the full path?

(A) (a) 0.32 ms; (b) 120 Mbps end-to-end (the fastest link)

(B) (a) 2 ms; (b) 8 Mbps end-to-end

(C) (a) 2 ms; (b) 50 Mbps end-to-end

(D) (a) 0.32 ms on the 50 Mbps link; (b) 8 Mbps end-to-end (the bottleneck)

Answer: D. Transmission on link 1 = 16000/50,000,000 = 0.32 ms. End-to-end throughput = min(50, 8, 120) = 8 Mbps regardless of the first link's own delay.

Q66. Direct full-mesh cabling is being compared to a centralized switch for 60 devices. How many cables does full mesh require, and how many MORE cables is that than the centralized-switch approach?

(A) 3,600 cables for full mesh, which is 3,540 more than a switch

(B) 120 cables for full mesh, which is 60 more than a switch

(C) 60 cables for full mesh, identical to a switch

(D) 1,770 cables for full mesh, which is 1,710 more than the 60 cables a switch would need

Answer: D. n(n-1)/2 = 60×59/2 = 1,770 for full mesh. A central switch needs only n = 60 cables. Difference = 1,770 − 60 = 1,710.

Q67. A 5,000-bit packet crosses a link with R = 2 Mbps and d = 5×10^5 m at s = 2.5×10^8 m/s. If R is then QUADRUPLED (to 8 Mbps) with everything else unchanged, what is the NEW total (transmission + propagation) delay?

(A) 2.625 ms (0.625 ms transmission + 2 ms propagation)

(B) 10.5 ms, because quadrupling R proportionally increases propagation too

(C) 5 ms (2.5 ms transmission + 2.5 ms propagation, unchanged from before)

(D) 0.625 ms total, since propagation becomes negligible

Answer: A. New transmission = 5000/8,000,000 = 0.625 ms. Propagation is unaffected by R: 500,000/250,000,000 = 2 ms. Total = 2.625 ms. Quadrupling R only reduces transmission delay — propagation is untouched.

Q68. A path has 3 links: 10 Mbps, 10 Mbps, and 10 Mbps (all equal). A rival path (used as backup) has links 5 Mbps, 40 Mbps, 40 Mbps. Which path has higher end-to-end throughput, and what is it?

(A) The rival (backup) path — 40 Mbps, since two of its three links are much faster

(B) The first (equal-rate) path — 10 Mbps versus the rival's bottleneck of 5 Mbps

(C) They're identical, since averages are the same

(D) The rival path — 5 Mbps is irrelevant because it's the shortest link, not the slowest

Answer: B. Throughput = min(all link rates). First path: min(10,10,10) = 10 Mbps. Rival path: min(5,40,40) = 5 Mbps. The uniform 10 Mbps path wins.

Q69. How many cables would a full-mesh network of 15 devices need, and if 5 more devices are added (to 20 total), how many ADDITIONAL cables does that require?

(A) 105 cables at 15 devices; 190 total at 20 devices, so 85 additional cables needed

(B) 15 cables at 15 devices; 20 at 20 devices, so 5 additional

(C) 105 cables at 15 devices; 100 at 20 devices (fewer, since larger networks are more efficient)

(D) 225 cables at 15 devices; 400 at 20, so 175 additional

Answer: A. 15×14/2 = 105. 20×19/2 = 190. Additional cables = 190 − 105 = 85 — illustrating the accelerating (quadratic) cost of adding devices to a full mesh.

Q70. A 12,800-bit video chunk must be sent over a satellite uplink: R = 2 Mbps, d = 3.6×10^7 m (geostationary altitude, one-way), s = 3×10^8 m/s. What is the ONE-WAY total delay (transmission + propagation), and roughly how much LARGER is propagation than transmission?

(A) Both are approximately equal at around 60 ms each

(B) Transmission = 120 ms, Propagation = 6.4 ms; transmission is roughly 19× larger

(C) Transmission = 6.4 ms, Propagation = 120 ms; propagation is roughly 19× larger

(D) Transmission = 6.4 ms, Propagation = 6.4 ms; they are identical

Answer: C. Transmission = 12800/2,000,000 = 6.4 ms. Propagation = 36,000,000/300,000,000 = 120 ms. Propagation dominates by roughly 120/6.4 ≈ 18.75×, reinforcing that satellite latency is a distance problem, not a bandwidth problem.

# Cross-Lecture Scenario Questions (Hardest)

Q71. Netflix buffers heavily at 10 PM. Engineers confirm: (1) all backbone links are under 40% utilization, (2) one origin server cluster is at 98% CPU, (3) users are on HTTP/1.1 still. Which combination of fixes targets the ACTUAL root cause FIRST, in priority order?

(A) Switch to HTTP/1.0 to reduce protocol overhead

(B) Add servers + load balancer (fixes the real server-side bottleneck) first; upgrading to HTTP/2 or HTTP/3 helps secondarily but doesn't fix an overloaded server

(C) Upgrade backbone link bandwidth first, since throughput is always link-limited

(D) Switch from packet switching to circuit switching to guarantee bandwidth

Answer: B. The bottleneck here is the server (98% CPU), not the links (40% utilization) or the HTTP version — load balancing across more servers is the direct fix; the HTTP version change is a secondary, unrelated improvement.

Q72. A packet traverses Network A → Network B → Network C. At the SAME time, its application data is protected end-to-end via TLS. Which statement correctly separates what changes hop-by-hop from what stays constant end-to-end?

(A) Layer 3 addressing is rebuilt at every hop while Layer 2 addressing persists end-to-end

(B) Both Layer 2 and Layer 3 addressing are rebuilt at every hop, while TLS decrypts the payload at each router for inspection

(C) Layer 2 (MAC) framing is rebuilt at every hop; Layer 3 (IP) addressing largely persists end-to-end; the TLS-encrypted application payload remains unreadable to every intermediate router regardless of hop

(D) TLS re-encrypts the payload differently at every hop, just like Layer 2 framing

Answer: C. Layer 2 is one-hop scope (rebuilt each time), Layer 3 addressing generally persists across the journey, and TLS's whole point is that intermediate routers CANNOT read the encrypted payload — encryption is end-to-end, not hop-by-hop.

Q73. An application is deployed in a single AZ within a Region, behind a load balancer, using HTTP/2 with a valid TLS certificate. A flood takes out that ONE AZ. Which specific design gap is responsible for the resulting downtime, given everything else was done correctly?

(A) Using HTTP/2 instead of HTTP/3, which caused the outage

(B) The load balancer itself caused the flood-related outage

(C) Not using asymmetric encryption for the actual video data

(D) Lack of a multi-AZ deployment — none of the load balancer, HTTP/2, or TLS setup protects against losing the only AZ hosting all the servers

Answer: D. Load balancing, HTTP version, and TLS operate at different concerns entirely; only multi-AZ deployment protects against a single data-center-level physical disaster like a flood.

Q74. A 500 Mbps satellite ISP plan still 'feels slow' for video calls, while a 50 Mbps fiber plan 'feels fast.' Using only concepts from Lecture 2, what is the most rigorous explanation?

(A) 500 Mbps is a marketing lie and the real speed is under 50 Mbps

(B) Video call latency is dominated by propagation delay (d/s), where satellite's huge distance far outweighs fiber's high speed advantage even at 10x lower bandwidth — bandwidth mainly affects transmission delay, which is small for either

(C) Bandwidth and latency are mathematically identical quantities

(D) Fiber always has zero propagation delay by definition

Answer: B. For interactive traffic like video calls, propagation delay (distance/speed) dominates perceived lag; satellite's huge round-trip distance overwhelms any bandwidth advantage, since bandwidth only shrinks the (already small) transmission-delay component.

Q75. A firewall rule (IP+Port based, Layer 3/4) is in place, and separately a TLS certificate secures the same server. An attacker with a stolen but VALID CA-signed certificate for a DIFFERENT legitimate domain tries to impersonate this server. What happens?

(A) TLS still fails — a valid certificate for domain X does not validate for domain Y, since certificate validation checks that the cert matches the exact requested domain, regardless of firewall rules

(B) The firewall alone blocks this since it's a Layer 3/4 device

(C) It succeeds because the certificate was issued by a trusted CA, regardless of domain match

(D) TLS is irrelevant here since the firewall already filters by IP and port

Answer: A. Certificate validation checks the certificate's exact domain binding, not just whether SOME trusted CA signed it — a valid cert for the wrong domain still fails validation. This is separate from (and not fixed by) firewall IP/port rules.

Q76. A REST API client sends a PATCH request that the server, due to a bug, processes as if it were a PUT (full replace). The client only included an 'email' field. What real-world consequence directly follows from the CRUD semantics discussed in Lecture 5?

(A) Nothing changes, since PATCH and PUT are functionally identical in the HTTP spec

(B) The server would reject the request outright due to a method mismatch error

(C) Only the email field could ever be affected regardless of which method semantics are used

(D) Other fields not included in the request (name, role, etc.) could be wiped out, since PUT means 'replace the entire resource' rather than 'update only what's given'

Answer: D. This tests the PUT-vs-PATCH semantic distinction: a bug that treats a partial payload as a full replacement (PUT semantics) risks silently deleting the resource's other fields.

Q77. A student argues: 'HTTP/3 makes TLS obsolete since QUIC already encrypts everything.' Evaluate this claim using concepts from Lectures 5 and 6.

(A) The claim is irrelevant because HTTP/3 doesn't use encryption of any kind

(B) The claim is fully correct — HTTP/3 requires no TLS handshake whatsoever

(C) The claim is correct because QUIC runs over TCP, which already handles all encryption

(D) The claim is misleading — QUIC actually integrates TLS 1.3 as part of its own handshake; TLS's certificate-based authentication and encryption goals aren't replaced, they're built directly into QUIC's transport-layer design

Answer: D. QUIC (HTTP/3's transport) integrates TLS 1.3 directly rather than eliminating it — encryption and certificate-based authentication remain essential; they're just built into the transport layer instead of layered separately on top of TCP.

Q78. A company's Star-topology office network goes down entirely when the central switch fails, while a neighboring smart-home's Mesh network stays functional when one smart bulb's hub dies. What single topology principle explains BOTH outcomes?

(A) Star topology's central switch is actually a benefit for fault tolerance, not a risk

(B) Both topologies have identical fault-tolerance profiles; the outcomes were coincidental

(C) Star has one single point of failure (the central device) affecting the whole network, while Mesh's redundant direct paths mean no single device failure isolates the rest of the network

(D) Mesh always requires fewer total connections than Star, which is why it's more reliable

Answer: C. This is precisely the Star (SPOF at the center) vs Mesh (high fault tolerance via redundant paths) trade-off from Lecture 4.

## Answer Key Distribution (for transparency — verify it's balanced)

A: 17   |   B: 18   |   C: 20   |   D: 23   |   Total: 78 questions